## Step 1: Use DC Motor Equation to Find Required Voltage (Va)

Why this step:
Now you know what torque the motor must produce. But your Arduino can only send voltage signals (as PWM). You need to figure out: "What voltage should I apply to the motor to produce -1.48 N·m of torque?"

The DC motor equation relates torque, voltage, and motor speed:

C=KmRVa-KmKeRω

Rearrange to solve for the voltage you need to apply:

Va=R⋅τ+KmKe⋅ωKm

Where:

- Va: voltage to apply to motor (Volts)

- τ: desired torque (N·m)

- ω: current motor angular velocity (rad/s) — measured from encoder

- Km: motor torque constant (N·m/A) from datasheet

- Ke: back EMF constant (V·s/rad) from datasheet

- R: armature resistance (Ω) from datasheet

Plain English:
"To produce -1.48 N·m of torque, given that the motor is currently spinning at 10 rad/s, I need to apply -8.5 Volts (approximately) to the motor."

Example Calculation:

Assume:

- Km=0.1 N·m/A (from motor datasheet)

- Ke=0.1 V·s/rad (from motor datasheet)

- R=1 Ω (from motor datasheet)

- ω=10 rad/s (measured from encoder)

- τ=-1.48 N·m (from Step 2)

Va=1×(-1.48)+0.1×0.1×100.1
Va=-1.48+0.10.1=-1.380.1=-13.8 V

text

V_a = -13.8 V (negative = backward)

## Step 2: Saturate Voltage to Available Supply Range

Why this step:
You calculated that you need -13.8 V, but your power supply might only deliver ±12 V (battery voltage). You can't apply more voltage than what's physically available!

Logic:

Valimited=clamp(Va,-Vmax,+Vmax)

Where:

- Vmax is the maximum available voltage from your battery

Plain English:
"I calculated -13.8 V, but I only have 12 V available. So I'll apply -12 V (the maximum possible)."

Example:

text

V_max = 12 V (battery supply)

V_a (limited) = clamp(-13.8, -12, +12) = -12 V

## Step 3: Convert Voltage to PWM Duty Cycle (0-255)

Why this step:
Arduino's analogWrite() function doesn't accept voltage values. It accepts integer values from 0 to 255, where:

- 0 = 0% duty cycle (motor off)

- 127 = 50% duty cycle (half power)

- 255 = 100% duty cycle (full power)

Mathematical Conversion:

PWM=∣ValimitedVmax∣×255

Where:

- Valimited: voltage after saturation (Volts)

- Vmax: maximum voltage your system can provide (Volts)

- The absolute value ensures PWM is always positive (0-255)

- The sign of Valimited determines motor direction

Plain English:
"I have -12 V and my max is 12 V. The proportion is -12/12 = -1. The magnitude is 1. So PWM = 1 × 255 = 255. And since it's negative, the motor goes backward."

Example:

text

PWM = |(-12) / 12| × 255 = 1 × 255 = 255

Direction = BACKWARD (because V_a was negative)

## Step 4: Apply PWM and Direction to Motor Driver

Why this step:
Most motor drivers (like DRV8825 for steppers or L298N for DC motors) have:

- A PWM pin to control speed (0-255)

- A direction pin to control direction (HIGH = forward, LOW = backward)

Plain English:
"Send PWM value of 255 (full speed) and set the direction pin to LOW (backward direction)."