Ziegler-Nichols Method: Step-by-Step Explanation

The Ziegler-Nichols method is a classical, empirical technique for tuning PID controller parameters to obtain a responsive, stable, and reasonably well-behaved system. There are two popular versions:

1. Step Response (Open-Loop) Method

2. Ultimate Gain (Closed-Loop) Method (you probably mean this one—most common in engineering control!)

Ultimate Gain (Closed-Loop) “Continuous Oscillation” Method

Step 1: Remove I and D Action

Set KI=0, KD=0. This leaves only the proportional gain (KP) active.

Step 2: Increase the Proportional Gain

Gradually increase KP from a low value until the system output oscillates indefinitely with constant amplitude (sustained periodic oscillation) but does not diverge or settle. This point is called the ultimate gain (Ku), and the period of the oscillation is called the ultimate period (Pu).

Record:

- Ku= ultimate gain (where oscillation just starts)

- Pu= period of oscillation

Step 3: Calculate the PID Parameters Ti (reset rate) and Td (rate)

| Controller Type | Kp | Ki (integral time, or Ti) | Kd (derivative time, or Td) |
| --- | --- | --- | --- |
| P | 0.5Ku | — | — |
| PI | 0.45Ku | Pu/1.2 | — |
| PID | 0.6Ku | Pu/2 | Pu/8 |

Step 4 :  Apply Ziegler-Nichols Formulas as per the below table

| Type | KP | KI | KD |
| --- | --- | --- | --- |
| P | 0.5Ku | — | — |
| PI | 0.45Ku | KP/Pu/1.2 | — |
| PID | 0.6Ku | 2KP/Pu | KPPu/8 |

Practical Example: PID Controller for Segway Transfer Function

Suppose you have already identified:

- Ku=80 (proportional gain at which system begins to oscillate)

- Pu=1.2 seconds (oscillation period)

PID Gains Calculation:

- KP=0.6×80=48

- Ti=Pu/2=1.2/2=0.6 seconds

- KI=KP/Ti=48/0.6=80

- Td=Pu/8=1.2/8=0.15 seconds

- KD=KP⋅Td=48×0.15=7.2

MATLAB Example Code

% Example plant transfer function for Segway:

num = [-166.67]; % Example numerator from previous answer

den = [0.00015 0 -133.33]; % Example denominator (Ks^2 - a), with a known sign

G = tf(num, den); % Create the plant transfer function

% Ziegler-Nichols tuning values (from procedure above)

Ku = 80;         % Ultimate gain (found from experiment)

Pu = 1.2;        % Ultimate period (seconds)

% PID parameters (Ziegler-Nichols)

Kp = 0.6 * Ku;             % Proportional gain

Ti = Pu / 2;               % Integral time

Td = Pu / 8;               % Derivative time

Ki = Kp / Ti;               % Integral gain

Kd = Kp * Td;               % Derivative gain

% Create MATLAB PID Controller

C = pid(Kp, Ki, Kd);

% Simulate closed-loop system

T = feedback(C * G, 1);   % Unity feedback

% Step response

figure;

step(T);

title('Closed-loop Step Response: PID Tuned by Ziegler-Nichols');

% Display gains

fprintf('PID gains (Ziegler-Nichols): Kp = %.2f, Ki = %.2f, Kd = %.2f\n', Kp, Ki, Kd);

Summary of Ziegler-Nichols Ultimate Gain Method

- Pro: Fast, experimental, widely used for industrial PID controller design.

- Con: Tends to result in "aggressive" PID parameters; may require manual fine-tuning for robustness/safety.

You can run the MATLAB code above after identifying Ku and Pu by gradually increasing the proportional gain in your simulation or hardware until you see constant amplitude oscillations.